| Global Laser Machine Vision & Inspection Development Kit |  |
| --- | --- |

- 660 nm, 35mW - User Adjustable Focus - Switched Power Supply 110/240 Vac to 5.0V Amp with UK, Euro & US plug - Wide variety of interchangeable optics supplied - Two year warranty - Case electrically isolated. The Global Laser Machine Vision Development Kit has been designed to meet the needs of system integrators and developers who have to solve often changing challenges in the machine vision and inspection markets. The Lyte-MV laser provides a reliable, high power, industrial laser light source with a well-defined line illumination. Used primarily with industrial cameras, they are suitable for a wide range of inspection, measurement and control systems. As a development kit, a heavy duty mounting clamp and a switched power supply is included. Laser positioning is made easy because the heavy duty mounting clamp which allows the laser to be adjusted and securely fixed at any required direction or angle as well as acting as a suitable heatsink. The switched power supply 110/240 Vac to 5.0V Amp is supplied with a UK, Euro & US plug. Also included are six easy interchangeable projection options: - Uniform Line Lens 45° - Cross Optic 37° - 7 Line Optic 21.7° - Projection Grid 4 by 4 4.57° - 5 Circles 28.6° - 21 by 21 Dot Array 7° The Development Kit is qualified to European specifications and are one of the most controllable lasers in the machine vision market. The Development Kit is provided with a rugged and attractive plastic carrying case designed to securely store all kit components.